Prof. Alexander Rohrbach, IMTEK

8. Tutorial to the lecture

Wave Optics SS 19

Solutions presented on 8.7.2019

Tutors: Rebecca Michiels, Tetiana Udod and Matthias Allkemper

Exercise 31 – Transmission grating (Exam level)

Sunlight falls onto a transmission grating with 5000 lines per cm.

- Do the diffraction orders m=+2 and m=+3 of the spectra overlap. Assume sunlight to have wavelengths between red ( = 780nm) and  violet ( =390nm)?

- Design a blazed structure of a transmission grating made of plastic (n = 1.50) and a reflection grating (5000 lines per cm) such that red is amplified in order m=+1.

Exercise 32 – Separation of sodium doublet lines (Exam level)

What is the minimum number of grating lines to separate the sodium doublet lines (1 = 589.0nm and 2 = 589.59nm) using the diffraction order m=+3.

Continuous spectrum of a light bulb. The orders m=+1 and m=+2 are visible on the right, while on the left only m=-1 is viewable.

Exercise 33 - Binary phase grating

Consider a binary phase grating in air generating  , where the grating period g is twice as broad as the  slit width b. The maximum phase step is m.

- Calculate the intensity spectrum distribution  of the diffracted light. Demonstrate that the following equation is valid:  |ei - 1|² = 2(1-cos).

- For which m the diffraction peaks have maximum intensity and what happens for m = 2?

- The grating is made of PMMA (n = 1.42) and it is optimized for a wavelength of  = 532nm. What is the height of the grating structure?

- What is the decrease in diffraction efficiency  (in %) if light with  = 633nm is used?

- Exercise 34 – Talbot effect  (Exam level)

An electric field distribution E(x,z=0) that is periodic in lateral direction generates a periodic electric field distribution E(x,z) in propagation direction z as well (see figure below).

Such a periodic light pattern with a spacing of z = 100µm in z-direction is projected into a  photosensitive polymer (refraction index n=1.40) using a mask with a lateral grating and light with a vacuum wavelength of  0 = 532nm.

- What is the lateral grating period of the mask?

- How can the shape of these recurring light patterns be changed without replacing the mask?

Re{E(x,y,z=0)}			Re{E(x,y=0,z)}				I(x,0,z)

Exercise 35 -  AOM (if topic already covered)

A laser beam ( = 633nm) is switched on and off with an AOM. The angle between the zeroth and first diffraction order has to be at least 5°, due to technical reasons. There are two crystals available: TeO2 (nk = 2.26, Vac = 4200 m/s) und SiO2 (nk = 1.46, Vac = 5960 m/s).

- Which of the two crystals would you chose to achieve low vibrational frequencies (the frequency of the acoustic wave)?

- What is the relative momentum transfer (in %) for the two crystals? Please, sketch the reciprocal grating.

- What is the change in laser frequency for the two crystals? (Exam level)

- Show how the AOM can achieve a diffraction angle of 5° for 532nm laser light using the TeO2 crystal.

- What happens if the frequency of the acoustic wave is varied for a constant laser wavelength ?